Table of Contents

1	Document Revision History	3

2	Purpose / Scope	3

3	References / Related Publications	3

4	Revision Histories	4

4.1	Revision:  1.0 (Prototype Release)	4

4.2	Revision:  1.1 (Prototype Release)	5

4.3	Revision:  1.2 (Production Release)	6

4.4	Revision:  1.3 (Production Release)	7

4.5	Revision:  1.4 (Production Release)	8

4.6	Revision:  1.5 (Development Release)	9

4.7	Revision:  1.6 (Production Release)	10

4.8	Revision:  1.7 (Production Release)	11

4.9	Revision:  1.8 (Production Release)	12

5	Open Document Issues	13

# Document Revision History

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| D-001 | 11/05/02 | KenK | Initial Draft to document all revision up to and including 1.06 |
| D-002 | 12/18/02 | KenK | Updated for revision 1.07 |

Table 1:  Document Revision History

# Purpose / Scope

The purpose of this Software Release Document (SRD) is to provide a mechanism to record the revision history for the firmware used in the Model 5130 Analog Input/Output Modules.  Within this document, each revision shall contain a list of:

- Known defects

- Corrected defects

- Enhancements

- Suggested enhancements for future releases

This specification is intended to be a living document.  It should be updated with all future firmware revisions, as well as with any additional information that is deemed necessary for the production and maintenance of the Model 5130 throughout its product lifecycle.  As defects and/or enhancements arise, they should be immediately added to the associated list so that they may be addressed in future releases.

For instructions on how to use any of the CTC software programs and/or the specifications of the hardware, the user should refer to the documents referenced in Section 3: References / Related Publications.

# References / Related Publications

- Hardware Release Document: 5130, Doc ID:  5130-HRD

- Data Sheet:  5100-A +/-10V Ains, Doc. ID:  5130-A-DS

- Data Sheet:  5100-K +/-100mV/Thermocouple Ains, Doc. ID:  5130-K-DS

- Data Sheet:  5100-Q +/-20mV Ains, Doc. ID:  5130-Q-DS

- Data Sheet:  5100-R 4-20mV Ains, Doc. ID:  5130-R-DS

# Revision Histories

## Revision:  1.0 (Prototype Release)

| Release Date: | 09/12/01 |
| --- | --- |
| Released By: | KenK |
| Filename: | 5130.hex |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 8k bytes EERPOM: 512 bytes SRAM: 512 bytes |
| Processor: | Atmel: AT908515-8AC |
| 5100 System Compatibility: | 5130 Hardware Revision: 0 only 5100 Hardware Revision: 0, A, B, C, E 5100 Firmware: Pre-release development code only |
| Release Usage: | Prototype release only. |
| Reasons for new release: | Initial revision for prototype modules. |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  | N/A | KenK | Channel-to-channel interference in the analog input circuits do to shared control signals. |
|  | N/A | KenK | Analog input readings are non-linear in the negative region due to the analog comparators used in the ADC circuitry. |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  1.1 (Prototype Release)

| Release Date: | 11/14/01 |
| --- | --- |
| Released By: | KenK |
| Filename: | 5130.hex |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 8k bytes EERPOM: 512 bytes SRAM: 512 bytes |
| Processor: | Atmel: AT908515-8AC |
| 5100 System Compatibility: | 5130 Hardware Revision: A only 5100 Hardware Revision: 0, A, B, C, E 5100 Firmware: Pre-release development code |
| Release Usage: | Prototype release only. |
| Reasons for new release: | Changed for revision A of the 5130 hardware. |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 11/14/01 | KenK | Fixed channel-to-channel interference in the analog inputs by adding an additional control signal. |
|  | 11/14/01 | KenK | An attempt to correct linearity issues with the analog comparators used in the ADC circuitry. |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  | N/A | KenK | Analog input readings are still non-linear in the negative region. |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  1.2 (Production Release)

| Release Date: | 12/06/01 |
| --- | --- |
| Released By: | KenK |
| Filename: | 5130.hex |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 8k bytes EERPOM: 512 bytes SRAM: 512 bytes |
| Processor: | Atmel: AT908515-8AC |
| 5100 System Compatibility: | 5130 Hardware Revision: B, C 5100 Hardware Revision: 0, A, B, C, E 5100 Firmware: 4.02 |
| Release Usage: | Full production release. |
| Reasons for new release: | Changed for revision B of the 5130 hardware. Optimized for reuse. |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 12/06/01 | KenK | Fixed non-linearity issues in the analog inputs by adding a real ADC. |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  | N/A | KevinF | SPI Communications errors that cause inconsistent results when attempting to read/write to the 5130’s SRAM. |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

Table 2:  Enhancements

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  1.3 (Production Release)

| Release Date: | 02/07/02 |
| --- | --- |
| Released By: | KenK |
| Filename: | 5130.hex |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 8k bytes EERPOM: 512 bytes SRAM: 512 bytes |
| Processor: | Atmel: AT908515-8AC |
| 5100 System Compatibility: | 5130 Hardware Revision: B, C 5100 Hardware Revision: 0, A, B, C, E 5100 Firmware: 4.02 |
| Release Usage: | Full production release. Upgrade all prior releases. |
| Reasons for new release: | Defect correction. |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 02/07/02 | KenK | Fixed timing issues in the SPI Communications errors that caused inconsistent results when attempting to read/write to the 5130’s SRAM. |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  1.4 (Production Release)

| Release Date: | 02/30/02 |
| --- | --- |
| Released By: | KenK |
| Filename: | 5130.hex |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 8k bytes EERPOM: 512 bytes SRAM: 512 bytes |
| Processor: | Atmel: AT908515-8AC |
| 5100 System Compatibility: | 5130 Hardware Revision: B, C 5100 Hardware Revision: 0, A, B, C, E 5100 Firmware: 4.02 |
| Release Usage: | Full production release Upgrade all prior releases. |
| Reasons for new release: | Optimized for reuse. |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |
|  | 02/30/02 | KenK | Optimized for reuse in the 2720 and future analog modules (i.e. major overhaul). |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  1.5 (Development Release)

| Release Date: | 04/03/02 |
| --- | --- |
| Released By: | KenK |
| Filename: | 5130.hex |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 8k bytes EERPOM: 512 bytes SRAM: 512 bytes |
| Processor: | Atmel: AT908515-8AC |
| 5100 System Compatibility: | 5130 Hardware Revision: B, C 5100 Hardware Revision: 0, A, B, C, E 5100 Firmware: 4.02 |
| Release Usage: | Development/Test release, not to be shipped. |
| Reasons for new release: | Standardization of the SPI handshaking protocols and packet framing. |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |
|  | 04/03/02 | KenK | Handshaking now done by toggling control lines when transmission is complete. |
|  | 04/03/02 | KenK | Packet formatting is now framed by the nSS control signal. |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  1.6 (Production Release)

| Release Date: | ??? |
| --- | --- |
| Released By: | KenK |
| Filename: | 5130.bin |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 8k bytes EERPOM: 512 bytes SRAM: 512 bytes |
| Processor: | Atmel: AT908515-8AC |
| 5100 System Compatibility: | 5130 Hardware Revision: B, C, D 5100 Hardware Revision: 0, A, B, C, E 5100 Firmware: 4.04.10, 4.04.11, 4.04.12 |
| Release Usage: | Full production release Upgrade all prior releases. |
| Reasons for new release: | Enhancements to the Status Register and assorted comments, as well as a new file format for 5100 In-System programming. |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |
|  | 05/09/02 | KenK | Added additional functionality to the Status Register. MSB: FF = Not Ready, 01 = Not Tuned, 00 = Ready LSB: EE error count SPDR = Not_Ready on power up. |
|  | 05/09/02 | KenK | Added comments pertaining to the SPI protocol. |
|  | 05/09/02 | KenK | Changed from Hex to Binary file format for 5100 In-System programming. |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |
|  | 11/04/02 | KenK | Code/Jumpers to allow the 5100 to distinguish between different assemblies of the 5130 (i.e. +/-10V, +/-20mV, +/- 100mV, and 4-20mA). This should be implement in a similar matter as in the 5131 and the 5132. |
|  | 11/04/02 | KenK | Possible processor upgrade to the ATMega32/3 for additional digital filtering capabilities. |

## Revision:  1.7 (Production Release)

| Release Date: | 12/19/2002 |
| --- | --- |
| Released By: | KenK |
| Filename: | 5130.bin |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 8k bytes EERPOM: 512 bytes SRAM: 512 bytes |
| Processor: | Atmel: AT908515-8AC |
| 5100 System Compatibility: | 5130 Hardware Revision: B, C, D 5100 Hardware Revision: 0, A, B, C, E 5100 Firmware: 4.04.13 |
| Release Usage: | Full production release Upgrade all prior releases. |
| Reasons for new release: | Enhancements to the Version register is distinguish between versions A, K, Q, and R PCA builds. |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  | 03/04/04 | KenK | Since there was no internal/external brownout circuitry on the original 5130, every now and then the AT90S8515’s EE would get scrambled. Solution: Change the processor to the ATMega8515, which has internal brownout circuitry. Note: This should not require a software change. |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |
|  | 11/04/02 | KenK | Added the Code/Jumpers to allow the 5100 to distinguish between different assemblies of the 5130: A = +/-10V K = +/-100mV Q = +/-20mV R = 4-20mA |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  1.8 (Production Release)

| Release Date: | 03/04/2004 |
| --- | --- |
| Released By: | KenK |
| Filename: | 5130.bin |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 8k bytes EERPOM: 512 bytes SRAM: 512 bytes |
| Processor: | Atmel: AT908515-8AC |
| 5100 System Compatibility: | 5130 Hardware Revision: B, C, D, E 5100 Hardware Revision: 0, A, B, C, E 5100 Firmware: 4.04.13 |
| Release Usage: | Full production release Upgrade all prior releases. |
| Reasons for new release: | Defect corrections. |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 03/04/04 | KenK | Since there was no internal/external brownout circuitry on the original 5130, every now and then the AT90S8515’s EE would get scrambled. Solution: Change the processor to the ATMega8515, which has internal brownout circuitry. Note: This should not require a software change. |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  | 09/01/04 | KenK | Incorrectly responses with a version of 1.09 instead of 1.08. |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |
|  | 03/04/04 | KenK | Modified EE read and write routines to always put the EEAR back to address 0000 when done. This should help protect the EE on power up. |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  1.9(Production Release)

| Release Date: | 09/01/2004 |
| --- | --- |
| Released By: | KenK |
| Filename: | 5130.bin |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 8k bytes EERPOM: 512 bytes SRAM: 512 bytes |
| Processor: | Atmel: AT908515-8AC |
| 5100 System Compatibility: | 5130 Hardware Revision: B, C, D, E 5100 Hardware Revision: 0, A, B, C, E 5100 Firmware: 4.04.13 |
| Release Usage: | Full production release Upgrade all prior releases. |
| Reasons for new release: | Defect corrections. |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 09/01/04 | KenK | No code changed. Just a correct to update the filename and the SRD to reflect V1.09. |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

# Open Document Issues

Table 3 is a compiled list of all open issues pertaining to this document and/or to the procedures described within.

| Issue # | Description |
| --- | --- |

Table 3:  Open Issues