Table of Contents

1	Document Revision History	3

2	Purpose / Scope	3

3	References / Related Publications	3

4	Revision Histories	4

4.1	Revision:  4.18 (Prototype Release)	4

4.2	Revision:  4.25 (Initial Release)	5

4.3	Revision:  4.28	6

4.4	Revision:  4.29	8

4.5	Revision:  4.30	9

4.6	Revision:  4.31	10

4.7	Revision:  4.32 (Unreleased)	11

4.8	Revision:  4.33 (Unreleased)	13

4.9	Revision:  4.34 (Unreleased)	15

4.10	Revision:  4.37	16

4.11	Revision:  4.38	17

4.12	Revision:  5.05	18

4.13	Revision:  5.06	19

5	Open Document Issues	20

# Document Revision History

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| B-001 | 12/19/02 | KenK | Initial Draft |
| C-002 | 04/02/03 | Stan Katz | First Official Release |
| C-003 | 08/25/04 | Stan Katz | Corrected problems, updated to GCC V3.2 compiler |
| C-004 | 09/02/04 | Stan Katz | Added Serial Number and Monitor Version parameters |
| C-007 | 08/17/05 | Stan Katz | Added information about Versions 4.32 and 4.33 |

Table 1:  Document Revision History

# Purpose / Scope

The purpose of this Software Release Document (SRD) is to provide a mechanism to record the revision history for the firmware used in the Model 5150 Dual Axis Stepper Module.  Within this document, each revision shall contain a list of:

- Known defects

- Corrected defects

- Enhancements

- Suggested enhancements for future releases

This specification is intended to be a living document.  It should be updated with all future firmware revisions, as well as with any additional information that is deemed necessary for the production and maintenance of the Model 5150 throughout its product lifecycle.  As defects and/or enhancements arise, they should be immediately added to the associated list so that they may be addressed in future releases.

For instructions on how to use any of the CTC software programs and/or the specifications of the hardware, the user should refer to the documents referenced in Section 3: References / Related Publications.

# References / Related Publications

- Hardware Release Document: 5150, Doc ID:  5150-HRD

- Data Sheet:  5100-TT, Dual Axis Servo , Doc. ID:  5150-TT-DS

# Revision Histories

## Revision:  4.18 (Prototype Release)

| Release Date: | 08/12/02 |
| --- | --- |
| Released By: | Stan Katz |
| Filename: | 5150.sr1 |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler sh-elf-gcc 2.95.2 (Microcross X-Tools 2.2c) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7045F28 |
| 5100 System Compatibility: | 5150 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.04.XX |
| Release Usage: | Prototype release. |
| Reasons for new release: | Prototype release |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  |  | Stan Katz | Communications with 5100 system fails |
|  |  | Kevin Ferrigno | Pulses may not all be generated during motion |
|  |  | Stan Katz | Frequent servo related errors in quickstep with 5100 V4.05.XX |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

Table 2:  Enhancements

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  4.25 (Initial Release)

| Release Date: | 04/02/03 |
| --- | --- |
| Released By: | Stan Katz |
| Filename: | 5150V0425.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 2.95.2 (Microcross X-Tools 2.2c) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7017F28 |
| 5100 System Compatibility: | 5140 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 |
| Release Usage: | Initial release. |
| Reasons for new release: | Initial release |

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 04/02/2003 | Stan Katz | Communications with 5100 system fails |
|  | 04/02/2003 | Ken Kollet | Pulses may not all be generated during motion |
|  | 04/02/2003 | Stan Katz | Frequent servo related errors in quickstep with 5100 V4.05.XX |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  | 04/02/2003 | Tim Sevener | Servo Profiling from registers containing large invalid numbers causes the 5140 to stop responding. |
|  | 12/12/2003 | Dawn Werner | Reprofiling with the velocity of zero stops any motion in progress. This is different to the 2219 behaviour. |
|  | 4/23/2004 | Dawn Werner | Changing to a lower speed while moving CCW uses the Acceleration value and not the Deceleration value |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

Table 3:  Enhancements

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  4.28

| Release Date: | 08/25/04 |
| --- | --- |
| Released By: | Stan Katz |
| Filename: | 5150V0428.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 3.2 (Microcross X-Tools 3.0) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7017F28 |
| 5100 System Compatibility: | 5150 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 |
| Release Usage: | All Production |
| Reasons for new release: | Corrected problems, revised to use current compiler |

The change was to keep the 5150 firmware matching the 5140 which is where the problem was reported, there are no customers seeing this problem on the 5150.

The major reason for the change was to get the 5150 to operate the same as the 2219 when the PROFILE command is used to pause a motion in progress. The 5150 was treating this as a SOFT STOP command and ending the current motion. The 2219 was simply ramping the velocity down to zero, but the status would indicate that the motion was still in progress and when the velocity was set to a non-zero value, the motion would continue to the original target position.

The 5150 Firmware was update to handle the “Motion Paused” state in which the current commanded velocity is zero but the axis is still busy with a motion. It also allowed the destination to be retained so that when the velocity is set to a non-zero value, the motion to the original target can resume.

The other change was that selecting whether to use the Acceleration or Deceleration rates when changing speed was considering the difference between the 2 speeds, and was selecting the incorrect rate when the speed was negative (CCW). This was revised to consider the difference between the absolute values of the speeds.

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 04/02/2003 | Tim Sevener | Servo Profiling from registers containing large invalid numbers causes the 5150 to stop responding. |
|  | 12/12/2003 | Dawn Werner | Reprofiling with the velocity of zero stops any motion in progress. This is different to the 2219 behaviour. |
|  | 4/23/2004 | Dawn Werner | Changing to a lower speed while moving CCW uses the Acceleration value and not the Deceleration value |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |
|  | 12/12/2003 | Stan Katz | Added the ability to pause motion by reprofiling with a zero velocity. The motion is suspended until a non-zero velocity is selected and then it continues to completion. The STOP command will correctly stop a Paused motion |
|  | 2/18/2004 | Stan Katz | Cosmetic changes to use gcc V3.2 compiler, mostly required adding a blank line at the end to avoid the compiler warning and matching printf long parameters to the format specifications. |

Table 4:  Enhancements

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  4.29

| Release Date: | 09/02/04 |
| --- | --- |
| Released By: | Stan Katz |
| Filename: | 5150V0429.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 3.2 (Microcross X-Tools 3.0) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7017F28 |
| 5100 System Compatibility: | 5150 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 5200 Firmware: 5.18 |
| Release Usage: | All 5200 Production |
| Reasons for new release: | Added ability to get Serial Number and Monitor Version for 5200 |

The major change is the revised Monitor that is installed with the software that allows the 5150 to be updated on a Synchronous SPI bus as well as the standard Asynch update, since the 5200 doesn’t have Asynch capability on the SPI bus. To make sure that the board is compatible with the 5200 the controller needs to detect and report the Monitor version.

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  |  |  | None reported. |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  |  |  | None reported. |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |
|  | 12/12/2003 | Stan Katz | Added ability to get the Serial number and the Monitor Version as additional Read Only parameters. |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  4.30

| Release Date: | 01/05/05 |
| --- | --- |
| Released By: | Stan Katz |
| Filename: | 5150V0430.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 3.2 (Microcross X-Tools 3.0) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7017F28 |
| 5100 System Compatibility: | 5140 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 5200 Firmware: 5.18, requires 5.30 to get improved performnce |
| Release Usage: | All 5200 Production |
| Reasons for new release: | Added ability to request that the controller monitor for events and indicate to the 5200 when the event has occurred |

The major change is the ability for the 5200 to request that the controller monitor for events and indicate to the 5200 when the event has occurred. This allows the controller to use a fast parallel read to determine if the event has occurred instead of needing a full SPI command. This has been used to significantly reduce the 5200 processor load and the SPI bus activity during the MONITOR SERVO STATUS Quickstep command.

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  |  |  | None reported. |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  |  |  | None reported. |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |
|  | 12/12/2003 | Stan Katz | Added ability to request that the controller monitor for events and indicate to the 5200 when the event has occurred. |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  4.31

| Release Date: | 02/11/05 |
| --- | --- |
| Released By: | Stan Katz |
| Filename: | 5150V0431.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 3.2 (Microcross X-Tools 3.0) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7044F28 |
| 5100 System Compatibility: | 5150 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 5200 Firmware: 5.18, requires 5.30 to get the performance improvement |
| Release Usage: | All 5200 Production |
| Reasons for new release: | Corrected problems with Positive EOT and random Acceleration spike at the end of the Acceleration ramp. |

This firmware will allow a Velocity move to move in a negative direction when the positive EOT input is active. There was a bug in the previous version that allowed a point to point move in a negative direction but wasn't allowing a velocity move because the axis wasn't moving in a negative direction (because it was not yet moving).

There was also a timing issue when the system switched between constant velocity and ramping motion. This could generate an invalid acceleration for several PID samples.

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 1/21/2004 | Dawn Warner | When the positive EOT input is active, a velocity move in the CCW direction will not work. The CCW point to point move can be used to move away from the EOT input. |
|  | 1/21/2004 | Dawn Warner | The system may generate a spike in acceleration at the end of the acceleration portion of a point to point move. |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  |  |  | None reported. |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  4.32 (Unreleased)

| Release Date: | Test Version – Not released |
| --- | --- |
| Released By: | N/A |
| Filename: | 5150V0432.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 3.2 (Microcross X-Tools 3.0) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7017F28 |
| 5100 System Compatibility: | 5150 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 5200 Firmware: 5.18, requires 5.30 to get the performance improvement |
| Release Usage: | All 5200 Production |
| Reasons for new release: | Corrected problems with Positive EOT and random Acceleration spike at the end of the Acceleration ramp. |

The change made in V4.28 that allows a move to be suspended by setting the maximum speed to zero was causing the system not to update the target position when the system stopped so that the move could be resumed, with the original destination, by setting the speed to a non-zero value. Since the system was using the soft stop routine to suspend the move when the velocity was set to zero, the soft stop command was also leaving the target unchanged.

The system has restored the code that updates the target position when motion stops but added a test to leave the target unchanged when the maximum speed is zero, since that is the special case where we need to keep the target.

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 4/12/2005 | Stan Katz | If the stop (soft) servo command is issued while the servo is still ramping to slew velocity, the servo motor appears to stop. But the servo1:stopped flag does not get set. The servo continues to move very slowly towards the commanded position (from the previous profile command). |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  | 4/21/2005 | Kevin Ferrigno | If a Home Sequence is interrupted by and ESTOP input and the servo is reset then running the home sequence again never finishes. It will move onto the home switch, but Register 14301 stays at 9, and the servo position in Register 14001 doesn't change. |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  4.33 (Unreleased)

| Release Date: | Test version – not released |
| --- | --- |
| Released By: | N/A |
| Filename: | 5150V0433.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 3.2 (Microcross X-Tools 3.0) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7017F28 |
| 5100 System Compatibility: | 5150 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 5200 Firmware: 5.18, requires 5.30 to get the performance improvement |
| Release Usage: | All 5200 Production |
| Reasons for new release: | Corrected problems with running the Home routine after the routine is interrupted by an EOT input. |

The ESTOP routine was not clearing the flags associated with Homing, so when the Home operation was restarted, the status flags were in an intermediate state and prevented the homing sequence completing. The routine has been updated to clear the appropriate flags when stopping the motor.

While I was working on this I noticed that I sometimes get a glitch (jump in position or fast movement to a new position) when I start the deceleration ramp on Soft Stop and I have added a couple of extra tests to make get rid of it (it was the same type of timing issue that we were getting at the end of the ramp).

The problem with the low velocity and high acceleration is caused by the calculation for the ramp distance returning a zero value (the value is less than 1 step). This means that the effective distance for the ramp segment is zero and the profile generator was ignoring the command since there was no motion during the segment. The routine was revised to always return a non-zero value for a valid ramp.

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 5/2/2005 | Stan Katz | If a Home Sequence is interrupted by and ESTOP input and the servo is reset then running the home sequence again never finish. It will move onto the home switch, but Register 14301 stays at 9, and the servo position in Register 14001 doesn't change. |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  | 4/29/2005 | Stan Katz | When the system is using a low velocity and a high acceleration, it sometimes fails to execute the motion. This can often happen during the Home sequence since the speeds are low and the default acceleration and deceleration values are being used. |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  4.34 (Unreleased)

| Release Date: | 1/6/2006 |
| --- | --- |
| Released By: | Stan Katz |
| Filename: | 5150V0434.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 3.2 (Microcross X-Tools 3.0) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7044F28 |
| 5100 System Compatibility: | 5150 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 5200 Firmware: 5.18, requires 5.30 to get the performance improvement |
| Release Usage: | All 5200 Production |
| Reasons for new release: | Corrected problems with running low velocity and high acceleration. |

The problem with the low velocity and high acceleration was corrected for continuous motion but was not fixed for point to point motion. This version has the changed made to the point to point motion routines as well.

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 8/19/2005 | Stan Katz | If a Home Sequence is interrupted by and ESTOP input and the servo is reset then running the home sequence again will never finish. It will move onto the home switch, but Register 14301 stays at 9, and the servo position in Register 14001 doesn't change. |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  | 11/30/2005 | Dawn Werner | Over a long period of time the actual motor position does not match the commanded position, the system appears to lose step pulses, |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |
|  | 12/20/2005 | Dawn Werner | Allow the backoff speeds for the Home sequence to be changed by the Quickstep program. When the system has a very high resolution encoder then the default speeds may be too slow to be useful. |

## Revision:  4.37

| Release Date: | 2/2/2006 |
| --- | --- |
| Released By: | N/A |
| Filename: | 5150V0437.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 3.2 (Microcross X-Tools 3.0) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7044F28 |
| 5100 System Compatibility: | 5150 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 5200 Firmware: 5.18, requires 5.30 to get the performance improvement |
| Release Usage: | All 5200 Production |
| Reasons for new release: | Corrected problems with Pulse generation – sometimes the system generated very short pulses that could not be handled by the external drive. Added the ability to set the backoff and index search speeds. |

Revised the software relating that generates the pulse output signals to make sure that the system was not generating very short step pulses. These pulses could not be handled by the external drive hardware and resulted in the motor position “Drifting” from the desired position over a long period of operation.

Added the ability to set the backoff and index search speeds by using the tens digit of the Home Direction register (17003 for the first axis). When the digit is zero then the default fixed speeds are used, when the digit is 1 to 9 then the speed of backoff from the Home input is 10% to 90% of the current maximum speed (as set by the PROFILE command) and the index search speed is the same ratio (10% to 90% corresponding to a tens digit 1 to 9) of the backoff speed.

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 1/6/2006 | Stan Katz | Changed the pulse generation code to avoid some race conditions that resulted in the generation of extremely short step pulses under some circumstances |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  |  | Stan Katz | Setting of the backoff speeds does not work properly when the value is negative |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |
|  | 12/20/2005 | Stan Katz | Added the ability to set the backoff and index search speeds by using the tens digit of the Home Direction register (17003 for the first axis). |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  4.38

| Release Date: | Not Released |
| --- | --- |
| Released By: | Stan Katz |
| Filename: | 5150V0438.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 3.2 (Microcross X-Tools 3.0) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7044F28 |
| 5100 System Compatibility: | 5150 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 5200 Firmware: 5.18, requires 5.30 to get the performance improvement |
| Release Usage: | All 5200 Production |
| Reasons for new release: | Corrected the handling of the backoff speed selection. |

Corrected the handling of the backoff speed selection when the value in the register is negative.

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 8/19/2005 | Stan Katz | Corrected the handling of the backoff speed selection when the value in the register is negative. |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  5.05

| Release Date: | Tested during April 2006, not released |
| --- | --- |
| Released By: | Stan Katz |
| Filename: | 5150V0505.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 3.4.0 (Microcross X-Tools 3.1) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7044F28 |
| 5100 System Compatibility: | 5150 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 5200 Firmware: 5.18, requires 5.30 to get the performance improvement |
| Release Usage: | All 5200 Production |
| Reasons for new release: | To maintain compatibility with the 5140 code which was ported to the new tools |

Upgraded to use the new compiler tools V3.4.0.

Added support for changing gains via Telnet on the 5200 and to log motion data at high sped

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |
|  | 4/11/2006 | C Southward | When executing a Quickstep sequence of commands to simulate Homing using an EOT input, the sequence of generating a Soft Stop command at the same time as the module is generating the automatic hard stop in response to the input can cause the system to get into a state where the status is busy but the axis is not moving |
|  | 4/12/2006 | Bob Dansereau | The system does not handle negative backoff values properly (recurrence of V4.37 problem, not fixed in the V5.x code) |
|  | 4/13/2006 | Stan Katz | When using the “Virtual Inputs” the input assignments are not cleared when the system is reset by software |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

## Revision:  5.06

| Release Date: | Testing started 4/13/2006, Released 5/4/2006 |
| --- | --- |
| Released By: | Stan Katz |
| Filename: | 5150V0506.sr / M150V0506.sr |
| Checksum: | N/A |
| Memory Requirements: | FLASH: 128k x 16 bytes EERPOM: 1k x 8 bytes SRAM: 256 x 16 bytes |
| Compiler/OS | GNU C Compiler: sh-elf-gcc 3.4.0 (Microcross X-Tools 3.1) OS: uCOS |
| Processor(s): | Hitachi SH2: HD64F7044F28 |
| 5100 System Compatibility: | 5150 Hardware Revision: A, B, C 5100 Hardware Revision: B, C, E 5100 Firmware: 4.05.32 5200 Firmware: 5.18, requires 5.30 to get the performance improvement |
| Release Usage: | All 5200 Production |
| Reasons for new release: | Corrected the handling of simultaneous Soft Stop and EOT commands. Clear input assignments on reset. |

Corrected the handling of simultaneous Soft Stop and EOT commands.

Added clearing of input assignments when resetting the channel or system.

### Correct Defects

| Item | Date | Corrected By | Issue |
| --- | --- | --- | --- |
|  | 4/13/2006 | Stan Katz | When executing a Quickstep sequence of commands to simulate Homing using an EOT input, the sequence of generating a Soft Stop command at the same time as the module is generating the automatic hard stop in response to the input is now handled correctly and updates the axis status correctly. |
|  | 4/13/2006 | Stan Katz | Corrected the handling of negative values in the Home Backoff register |
|  | 4/13/2006 | Stan Katz | Clear all input assignments when resetting a channel |

### Known Defects

| Item | Date | Found By | Issue |
| --- | --- | --- | --- |

### Enhancements

| Item | Date | Implemented By | Issue |
| --- | --- | --- | --- |

### Suggested Enhancements

| Item | Date | Suggested By | Issue |
| --- | --- | --- | --- |

# Open Document Issues

Table 5 is a compiled list of all open issues pertaining to this document and/or to the procedures described within.

| Issue # | Description |
| --- | --- |

Table 5:  Open Issues